TrackPlot User Manual

TrackPlot is a software package designed to support the processing and interpretation of data from the class of marine animal tags that contain 3-axis accelerometers and 3-axis magnetometers as well as a pressure sensor.  DTAG and Acousonde are currently supported.

The main purpose of TrackPlot is providing a visual interface that allows for the rapid identification, illustration and quantification of kinematic patterns, where a kinematic pattern is defined as a short time series of animal attitudes and depth information.

TrackPlot produces a pseudo-track from the raw data and almost all user interaction is done with reference to some point on the pseudo-track.  Note that it is called a “pseudo-track” rather than simple a “track” because the sensor package does not measure position or speed through the water and so only a rough approximation to the animal’s trajectory through the water can be created. Trackplot can create new pseudo-tracks from raw tag data and it can georeference the track if a series of fixes exists, and it can be used to analyze pseudo-tracks that have already been created.

# User Interface

To first learn about the TrackPlot interface it is easiest to start with a track that has already been created.  A sample track file is provided with your installation package.

To get started, double click on the TrackPlot program. It will ask for the name of a config file in a text window. Type in the name of the file (e.g. w189b) and hit return. Two additional windows should appear, one containing a trackPlot track view and a second containing an interface panel.

Figure 1. shows the main screen and the on-screen control widgets. The user interface is track centric.  The view is centered on a specific point on the track called the currentPoint that can be advanced or moved back and forwards along the track.

## Rotations about the currentPoint

To rotate the track to get a different viewing angle hold down the left mouse button with the cursor somewhere near the middle of the display window.

Moving the mouse up or down changes the pitch angle of the view direction

Moving the mouse left or right rotates the scene about and axis defined by the depth scale.

## Moving forwards and backwards

Traversing backwards and forwards along the track is accomplished by means of the track advance control widget shown in Figure 1. The distance from the widget center determines the distance moved on a single left mouse click.  Holding down the button causes continuous motion either forward or backward. The fastest forward motion is achieved at the far right. With every movement the time since tag on in seconds and the time of day and the depth in meters is updated in the text window.

## Zooming in and out

The scale control widget zooms in + and out - while keeping the currentPoint at the center of the screen.

# Figure 1. TrackPlot screen showing some of the main features of the interface.

## Animation

Use the on-screen Play button to turn animation off and on.  This causes the animal to continously advance along the track.  When in animation mode the Track Advance control can be used to adjust the animation speed.

| # Figure 2. The control panel is used to select a variety of viewing and statistical options. _______________________________ # Track View Options The Top part of the Control Panel is used to set various viewing options. A model of the animal can be shown on the track if a DXF model is available using the Show Whale button. The Show Fluking option is designed to make it clear where fluke strokes occur. The Narrow Track option causes a narrower track to be drawn. This is useful when motion patterns are very dense. The Show Grid option positions a 1Km square under the track to give a sense of scale. The Hide Controls option hides the on screen controls. This is useful when saving images of track sections The Show Markers option causes markers to be shown or hidden if a marker file is provided via the config file. |  |
| --- | --- |

## Showing fluke strokes

The fluke stroke visualization is based on the second derivative of the pitch about a lateral axis (in animal coordinates). This often gives a useful indication of fluke strokes.   When selected the derivative values are displayed using a saw tooth pattern along the spine of the pseudo-track ribbon. The effectiveness of this feature, however, depends critically both on the placement of the tag on the animal and on the amount of smoothing that has been applied, either during track creation from the raw tracks.  For humpback whales for example, smoothing so that the point to point interval is approximately 0.8 seconds has proven to be very effective.

# Output File Box

The output file box can be used to enter a file name either for statistics files or for snapshots of the visualization.

## Save Image

The save image button causes the current screen to be saved as a tif image.  In addition to a full size image a quarter size version is also saved. This can be useful for saving a large number of features.

## Save File

The Save  File button causes the current file to be closed and a new file to be opened. If you are saving points or sections it will have an incremented number in the file name.

# Plot Options

Various time series plots can optionally be displayed in the main window to assist in track analysis.  All except for the spectrogram are updated in real-time.

## Plot Vertical Velocity

This plot shows the change in depth over time as recorded by the pressure sensor.

## Plot Frequency Pitch

This plot shows the result of a fourier analysis of the pitch angular accelerations for small section of track centered on the trackCenter.  The scale gives wavelengths in seconds.  The centroid frequency is shown above the plot.

## Show Frequency Acceleration

This plot shows the result of a fourier analysis of a the depth accelerations for small section of track centered on the trackCenter.  The scale gives wavelengths in seconds.  The centroid frequency is shown above the plot.

## Show Vertical Acceleration

This plot shows the length of the accelerometer vector after gravity has been subtracted.  This value is far more sensitive to vertical accelerations than horizontal accelerations.

## Sound Spectrogram

If sound files are available then it is possible to plot an acoustic spectrogram.  An example is shown in Figure 3. When showing spectrograms the Refresh button must be used at every new location.  Whenever a sound selection is refreshed it will play through your See the file setup section of this manual for information on how to specify sound files.

## Spect Speed

This shows a speed plot overlaid on the sound levels at low frequencies. This must be based on a calibration of the relationship between speed and sound energy level.

# Statistics Rollout

| Figure 2. The Statistics rollout panel allows for various statistics to be saved to files. |  |
| --- | --- |

Various options are provided for saving sections of track for later analysis or to create more conventional plots. The default output files are named sample1.txt, sample2.txt, sample3.txt, etc. Alternatively a filename can be entered using the output file box (remember to hit the Enter key after inputting a file name).

There are different kinds of information that can be saved, selectable via the control panel.

Points Mode: Use the ‘r’ key to save information about a point on the track to a data file. The following values are recorded.

Dive number: an integer dive number counting from the first dive after tagon.

Dive start: the HMS start time.

Dive length:  dive length in seconds.

Deepest: the deepest point in the current dive.

Time hms: the HMS time of the current point

Time sec: the time in seconds since tag on for the current point.

Depth: the depth of the current point.

Roll, Pitch and Heading: for the current point.

In addition, if a classify file exists a classification for the current point is added.

Sections Mode: save sections of track. All values decimal time, tagonsec, azimuth, pitch, roll, depth, azimuthal turn and rate pitch rate are recorded in a file from setPoint to trackCenter.

Select Save Sections mode from the panel

Enter a file name in the Output File box and hit return (otherwise default names will be used).  You can either use the ‘s’ and ‘r’keys to save the start and end of a section, or use the screen buttons as follows.

Use the Set button to set the start of a track section. Move to the end of the section and

Hit Rec to store  a section of track. Repeating will store more sections in the same file.

To close the file use the Save File button on the panel.

Histogram mode: computes and saves a specified set of histograms for a selected section of the track. These can be specified using the pattern recorder rollout panel.

# Creating a Pseudotrack From Raw Data

In order to create a track from raw data the elements a special configuration file must be set up having a .txt suffix.

| # Figure 3. An example contents of a text file used to define parameters necessary to construct a pseudo-track from raw data. | raw_smooth 4 speed 1.0 bin_file mn09_121a.bin tagon_hms 8 42 03 animal HUMPBACK fix_file Fixes_mn121a.txt name mn121a end |
| --- | --- |

raw_smooth  #  :  this parameter defines the amount of smoothing that is done as the raw data is loaded.  Smoothing is done using a Hamming filer.  If a value of 4 is given and the raw tag data is sampled at 5 Hz the result is a track with 0.8 sec time steps.  Generally more smoothing is better for a large slow moving animal such as a whale and less smoothing is desirable for a fast moving animal such as a dolphin.  Note If DTAG prh files have been used the “raw” data has already been downsampled to 5 Hz.

raw_sample # : This parameter defined the input sample rate. In the case of Acousonde it is user settable.  10 Hz is usually a good choice.

Speed # : this parameter defines the assumed speed of the animal through the water. This is used if the animal type has not been specified.

Bin_file <fname> :  this specified the name of the raw binary data used to construct the track.  In the case of DTAG a windows utility exists to combine files into a Bin_file.

animal HUMPBACK | MANATEE : (optional) if this parameter is given a simple model can be applied to add speed to the track.  This model has not been scientifically validated but it will certainly result in a speed profile that more accurately match the actual speed of the animal through the water.  The model

fix_file <fname> : (optional) If a set of surface fixes are available in UTM coordinates, an attempt can be made to fit the pseudo-track to the fixes. The Presence of strong and/or variable currents will generally result make it impossible to produce a satisfactory result.

name : (optional) The name of the animal.

end : This marks the end of the information read by the computer.  Notes can be added after this point in the file.

| # Figure 4. The rollout panel for creating a new track starting with raw tag data. |  |
| --- | --- |

The main difficulty in creating a pseudo-track is correctly situating the tag on the animal. First expand the Create New Track rollout menu shown in Figure 4. Situating the tag on the animal is actually best thought of as situating the animal with respect to the tag.

## Step 1.  Setting horizontal

The purpose to this step is to orient the animal with respect to the tag so that when it was actually horizontal it appears so on the track.

- Use the track advance widget to move to a section of track where you expect the animal to be horizontal (dorsal up, ventral down), for example, on the surface.  Use the screen Rec   button to make an initial track.  A ribbon psedudotrack should appear.

- Next move along the track looking for at least a few seconds of horizontal swimming.  At the start of the horizontal section set a point using Set Next move a short distance along the track and hit the Rec   button.  This causes all the gravity vectors between the two points to be averaged and this defines the down direction for that animal when it is horizontal.

- If successful the result is a track ribbon. The track should be horizontal at points where you expect the animal to be in a normal horizontal position.  Step 2 can be repeated if the track does not look right.

The track will likely still appear kinked for dives and ascents because heading has not been set.

| # Figure 5a. |  |
| --- | --- |
| # Figure 5b. |  |

## Step 2. Setting heading

The most effective way of defining heading is with a steep dive.  The reference frame is used for this (Figure xx).

- Position the trackCenter in the middle of a steep dive or steep ascent.  Get a sideways on viewpoint.

- Use the rotation widget to rotate the frame so that the blue ball is pointing down (if the animal is descending) as much as possible and so that the green side arm is horizontal. If this animal is ascending the blue ball should be pointing up.  It important to rotate the view to a side-on orientation.

- Click the setAz button.  You have just created a pseudo track.

Steps 1 and 2 can be repeated as many times as necessary until a satisfactory track is obtained. Once this has been achieved check the □ Track Set   box on the Interface Panel rollout.

Tags often change position on the animal and to deal with this it is necessary to repeat the setting process at the location on the track where the change is detected.  Detecting a shifted tag is done by noting where the animal appears to have changed its attitude, or the track may look skewed.   To correct of a shift in the tag first get to a point on the track after the shift, the set horizontal as before. Next move backwards along the track to get to the first steep ascent or descent and apply step 2 to set a new heading.  The changes are applied forward in time from this point.

If there are many changes in tag orientation the process can be repeated as necessary along the length of the track.  Note that detecting tag movements is a tricky subjective process. It may be necessary to start over several times before a satisfactory track is obtained.

Finally, check the save track button to produce a .vec file.  The filename of the .vec file should normally be changed to make it unique.  This filename will be included in the config file.

## Step 3 Creating a georeferenced track.

TrackPlot has features to tie the pseudo-track to a set of georeference points in a file consisting of a list of utm coordinates, and time values.  The process of making a georeferenced pseudo-track has many pitfalls, and the result are not guaranteed to be good.  Factors such as the presence of water currents cannot be taken into account and for this reason it may not be possible to create a very satisfactory pseudo-track even with good fixes..  The un-georeferenced pseudo-track is often best for interpreting behavior.

TrackPlot uses information about fluking together with fixes as an input to a three parameter model. A fourier analysis of fluking indicators is used to drive the animal  forward. The animal is slowed by drag proportional to the square of the speed.

The algorithm is as follows:

- First go through the normal process  to position the tag with respect to the whale.

The model is as follows.

Speed = speed + flukingEnergy*C1 – speed*c2 – speed*speed*C3.

Starting parameters are provided based on previous data from that kind of animal and  the parameters are tuned using an optimization algorithm that is run to create the best fit with the whatever fixes are available.

# Support for Behavior Classification

Trackplot has two kinds of support for behavior classification.  The first is an interface that supports the coding of kinematic events. Based on examination of the track, the scientist must define a set of kinematic categories (see file section).  If this category file has been provided a rollout on the interface panel appears enabling the time and properties of a particular kinematic patter to be saved.

# All Dives

When TrackPlot loads a track it automatically computes a set of dive statistics and puts them out in a file called AllDivesStats.csv . This can be opened by Excel.  Generally it is a good idea to rename this file before opening it.  The problem is that Excel can cause TrackPlot to crash if it has AllDivesStats open already.

AlldivesStats contains the start time of each dive,  as well as the start and end in seconds since tag on.  It contains the dive duration and the duration of the surface time following the dive. It also contains the maximum depth, the speed calculated from the dive path length, and the speed calculated by the surface distance between the dive location and the surfacing location.  Finally it contains the average turn rate for each dive in degrees/second.  Average turn rate is computed by taking the angular difference between each caudal vector and the next. This takes into account both azimuthal turns and pitch turns.

# Files

TrackPlot uses various file as information inputs

## Track File (.vec or .gzx)

These files contain the actual track information in the form of a series of attitudes and positions. A gzx format specifies the older files. The newer .vec format files store the attitude information in the form of dorsal and rostral vectors. In addition .vec contains acceleration information.

## Config file

This is the file that must be specified when the program starts.  It can be used to define a number of program parameters

| Figure 6. An example of the contents of a config text file used to define parameters necessary to construct a pseudo-track from raw data. | smooth 40 bin_file mn09_121a.bin tagon_hms 8 42 03 animal HUMPBACK fix_file Fixes_mn121a.txt name mn121a end |
| --- | --- |

smooth #  :  this parameter allows for additional smoothing that is done as the raw data is loaded. It is only used for the old-style .gzx files. Smoothing is done using a Hamming filter.

vec_file <fname> :  This specifies the track file e.g. mn152a.vec.  Older data uses the gzx format.

animal HUMPBACK | MANATEE : (optional)  This is used to specify which model is used to display the animal.  The default model is a humpback whale.

fix_file <fname>: Fixes_mn121a.txt If a set of surface fixes are available in UTM coordinates, a georeferenced track can be specified.

spectrogram <fname>: If a set of sound files are specified then a spectrogram of the sound can be displayed as one of the plot options.  Sound files are so large that it is often convienient to store them on an external drive in which case a full path must be specified. E.g. F:\mn09_127a\mn127a

name : The name of the animal.  This is used to construct the names of some of the output files.

Marker_file <fname>: (optional) optional: a file containing a set of time referenced values. These appear as color coded points along the track.  The option is useful for integrating information that has been obtained through external analysis using other software tools.

end : This marks the end of the information read by the computer.  Notes can be added after this point in the file.

## Shortcut keys

A number of shortcut keys are used for more rapid navigation through the file.

]		move forward 20 steps.

[		move backward 20 steps.

<shift>[		move forward 1 step.

<shift>]		move backward 1 step.

<ctrl >[		move forward 200 steps.

< ctrl >]		move backward 200 steps.

,		zoom in.

.		zoom out.

## Other magic keys

g	print a georeferenced track.  This prints a file containing time Lat, Lon and depth points. The points are decimated by a factor of 5.

+/-	Increase/decrease the sound volume

## Movies

The more recent version of TrackPlot let your record a sequence of video frames by pressing the 'v' key while the animal is moving along the track.  This saves a sequence of tif images in the Frames directory (you must have a Frames directory).  Use Apple Quicktime Pro with the "open image sequence" option to assemble the images into a video.  Once the sequence has been loaded into Quicktime you can save it as an avi or mpeg4 video.  Other software also lets you do this.

# Other features

TrackPlot has a number of additional features hidden in the code.  Contact Colin Ware if you are interested in possibilities for further development.

TrackPlot has been developed by Colin Ware with advice on requirements from a number of scientists.  In particular, comments encouragement and suggestions from David Wiley have proven invaluable. Roland Arsenault provided assistance with coding certain features and with setting up the environment for cross platform compilation. TrackPlot was initially developed to support the interpretation of data from DTAG tagged Humback Whales as part of a Stellwagen Bank National Marine Sanctuary’s Project (Chief Scientist: David Wiley).  Later refinements have been made under and ONR grant to Colin Ware [Grant #N0014091601].

Contact cware@ccom.unh.edu